UNIT - I

BASIC MEASUREMENT CONCEPTS

PERMANENT MAGNET MOVING COIL INSTRUMENT (PMMC)

- When a current carrying coil is placed in the magnetic field produced by permanent magnet, the coil experience a force and moves.

- This basic principle is called D’Arsonval Principle

- Iron core-Spherical if coil –circular

- Iron core-Cylindrical if coil-Rectangular

- Torque Equation

Td=NBAI

Td-Deflection torque in N-M

B-Flux Density, wb/m^2

N-Number of turns of coil

A-Coil area, m^2

I-Current

G=NBA=constant

Td= GI

The Controlling torque,

Tc=Kθ

For Steady state

Td=TC

θ=(G/K)I

Advantages, Disadvantages and Errors in PMMC

- Advantages:

- Sensitivity is high

- Consumes Low power

- Disadvantages:

- Cost is high

- Errors:

Source of errors are

- Friction

- Temperature

- Ageing of various parts

MULTIMETER

- A multimeter or a multitester, also known as a volt/ohm meter or VOM, is an electronic measuring instrument that combines several functions in one unit.

- A standard features for multimeters is the ability to measure voltage, current and resistance. There are two categories of multimeters, analog multimeters and digital multimeters (often abbreviated DMM.)

- A multimeter can be a hand-held device useful for basic fault finding and field service work or a bench instrument which can measure to a very high degree of accuracy.

- They can be used to troubleshoot electrical problems in a wide array of household devices such as possibly dead batteries, washing and drying machines, kitchen appliances, and automobile electronics and electrical systems.

WEIN BRIDGE

- Used for Frequency measurement.

- Z1=R1-j/ωC1

- Y3=1/R3+jωC3

- Z2/Y3=Z1Z4

- Equating real and imaginary parts

f=1/(2∏√ C1C3R1R3)

- If R1=R3 & C1=C3

f=1/(2∏RC)

MAXWELL BRIDGE

- Used for Inductance measurement.

- Y1=1/R1+jωC1

- Zx=Rx+jωLx, Z2=R2, Z3=R3

- General equation

Zx=Z2Z3Y1

- Equating Real and imaginary parts

Rx=R2R3/R1

Lx=R2R3C1

WAGNER GROUND CONNECTION

- Stray capacitance are external

capacitance that shunt the bridge

arms and cause measurement

errors at high frequencies.

- The oscillator is removed its

ground connection and bridged

by a series combination of resistor

and capacitor

- The junction of Resistor and

capacitor is grounded and this is

called the wagner ground

connection

UNIT II     BASIC ELECTRONIC MEASUREMENT

CATHODE RAY OSCILLOSCOPE

- The earliest and simplest type of oscilloscope consisted of a cathode ray tube, a vertical amplifier, a timebase, a horizontal amplifier and a power supply. These are now called 'analogue' scopes to distinguish them from the 'digital' scopes that became common in the 1990s and 2000s.

- Before the introduction of the CRO in its current form, the cathode ray tube had already been in use as a measuring device. The cathode ray tube is an evacuated glass envelope, similar to that in a black-and-white television set, with its flat face covered in a phosphorescent material (the phosphor).

- In the neck of the tube is an electron gun, which is a heated metal plate with a wire mesh (the grid) in front of it. A small grid potential is used to block electrons from being accelerated when the electron beam needs to be turned off, as during sweep retrace or when no trigger events occur.

CATHODE RAY TUBE

1. Electron guns 2. Electron beams 3. Focusing coils 4. Deflection coils 5. Anode connection 6. Mask for separating beams for red, green, and blue part of displayed image 7. Phosphor layer with red, green, and blue zones 8. Close-up of the phosphor-coated inner side of the

- CATHODE-RAY TUBE

- Power and Scale Illumination:  Turns instrument on and controls illumination of the graticule.

- Focus:  Focus the spot or trace on the screen.

- Intensity:  Regulates the brightness of the spot or trace.

- VERTICAL AMPLIFIER SECTION

- Position:  Controls vertical positioning of oscilloscope display.

- Sensitivity:  Selects the sensitivity of the vertical amplifier in calibrated steps.

- Variable Sensitivity:  Provides a continuous range of sensitivities between the calibrated steps. Normally the sensitivity is calibrated only when the variable knob is in the fully clockwise position.

- HORIZONTAL-SWEEP SECTION

- Sweep time/cm:  Selects desired sweep rate from calibrated steps or admits external signal to horizontal amplifier.

- Sweep time/cm Variable:  Provides continuously variable sweep rates. Calibrated position is fully clockwise.

- Position:  Controls horizontal position of trace on screen.

- Horizontal Variable:  Controls the attenuation (reduction) of signal applied to horizontal aplifier through Ext. Horiz. connector.

- TRIGGER

- The trigger selects the timing of the beginning of the horizontal sweep.

- Slope:  Selects whether triggering occurs on an increasing (+) or decreasing (-) portion of trigger signal.

- Coupling:  Selects whether triggering occurs at a specific dc or ac level.

- CONNECTIONS FOR THE OSCILLOSCOPE

- Vertical Input:  A pair of jacks for connecting the signal under study to the Y (or vertical) amplifier. The lower jack is grounded to the case.

- Horizontal Input:  A pair of jacks for connecting an external signal to the horizontal amplifier. The lower terminal is graounted to the case of the oscilloscope.

- External Tigger Input:  Input connector for external trigger signal.

ELECTRON GUN

- An electron gun is a component that produces an electron beam that has a precise kinetic energy, being used in televisions and monitors which use cathode ray tube technology, and in other instruments, as electron microscopes and particle accelerators.

FLUORESCENT SCREEN

Fluorescence is a luminescence that is mostly found as an optical phenomenon in cold bodies, in which the molecular absorption of a photon triggers the emission of another photon with a longer wavelength. The energy difference between the absorbed and emitted photons ends up as molecular vibrations or heat

Dual beam oscilloscope

- A dual beam oscilloscope was a type of oscilloscope once used to compare one signal with another. There were two beams produced in special type of CRT. Unlike an ordinary "dual-trace" oscilloscope (which time-shared a single electron beam, thus losing about 50% of each signal).

- Two pairs of vertical plates deflected the beams. Vertical plates for channel A had no effect on channel B beam. Similarly for channel B, separate vertical plates existed which deflected the beam B only.

Analogue storage oscilloscope

- An extra feature available on some analogue scopes is called 'storage'. This feature allows the trace pattern that normally decays in a fraction of a second to remain on the screen for several minutes or longer. An electrical circuit can then be deliberately activated to store and erase the trace on the screen.

- The storage is accomplished using the principle of secondary emission. When the ordinary writing electron beam passes a point on the phosphor surface, not only does it momentarily cause the phosphor to illuminate, but the kinetic energy of the electron beam knocks other electrons loose from the phosphor surface.

Digital Storage Oscilloscope

- The digital storage oscilloscope, or DSO for short, is now the preferred type for most industrial applications, although simple analogue CROs are still used by hobbyists. It replaces the unreliable storage method used in analogue storage scopes with digital memory.

RF POWER MEASUREMENT

- Resistors dissipate energy in the form of heat, and the rate at which they dissipate energy is called power. The power dissipated by resistors is delivered by the voltage and/or current sources in the circuit.

- The unit of power is the watt (one joule/second).

- There are several ways of calculating the power of resistors.

Power in A.C. Cicuits

- Instantaneous power:  p(t) is the time function of the power, It is the product of the time functions of the voltage and current. This definition of instantaneous power is valid for signals of any waveform. The unit for instantaneous power is VA

- Reactive power:  Q is the imaginary part of the complex power. It is given in units of volt-amperes reactive (VAR). Reactive power is positive in an inductive circuit and negative in a capacitive circuit.

- Apparent power: S is the product of the rms values of the voltage and the current, S = U*I. The unit of apparent power is VA. The apparent power is the absolute value of the complex power, so it is defined only for sinusoidal excitation.

- Power  Factor  (cos φ)

The power factor is very important in power systems because it indicates how closely the effective power equals the apparent power. Power factors near one are desirable. The definition:

Q-METER

- Terminals are provided to connect the inductance (Lx) to be measured and this is resonated by a variable tuning capacitor (C). Terminals are also provided to add capacitance (Cx), if required.

- The tuned circuit is excited from a tunable signal source which develops voltage across a resistor in series with the tuned circuit. A resistance of a mere fraction of an ohm is necessary. Metering is provided to measure the AC injection voltage across the series resistor and the AC output voltage across the terminals of the tuning capacitor.

- The tuning capacitor (C) of the Q meter has a calibrated dial marked in pico-farads, the value of inductance (Lx) can be derived. The tuned circuit is simply set to resonance by adjusting the frequency and/or the tuning capacitor for a peak in the output voltage meter

Lx = 1/4π²f²C

UNIT – III

SIGNAL GENERATORS AND ANALYZERS

FUNCTION GENERATORS

A function generator is a piece of electronic test equipment or software used to generate electrical waveforms. These waveforms can be either repetitive, or single-shot in which case some kind of triggering source is required (internal or external).

SIGNAL GENERATOR

- A signal generator, is an electronic device that generates repeating electronic signals (in either the analog or digital domains). They are generally used in designing, testing, troubleshooting, and repairing electronic or electroacoustic devices

SWEEP GENERATOR

- A low-frequency sawtooth wave is generated from some form of oscillator or waveform generator.

- Over a single sweep of frequency, RF output voltage from the device, as a function of time, is a plot of the filter response.

- By synchronising the sweep of the CRO with the sawtooth output, the device response is plotted on the CRO screen.